VLAN com DHCP

1º Configurando as VLAN no switch

Switch>en

Switch#config t

Switch#config terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#no ip domain-lookup

Switch(config)#vlan 2

Switch(config-vlan)#name ADM

Switch(config-vlan)#exit

Switch(config)#vlan 3

Switch(config-vlan)#name CONTABIL

Switch(config-vlan)#exit

Switch(config)#vlan 4

Switch(config-vlan)#name TI

Switch(config-vlan)#exit

Switch(config)#int fa0/2

Switch(config-if)#sw

Switch(config-if)#switchport ac

Switch(config-if)#switchport access vl

Switch(config-if)#switchport access vlan 2

Switch(config-if)#int fa0/3

Switch(config-if)#switchport access vlan 2

Switch(config-if)#int fa0/4

Switch(config-if)#switchport access vlan 2

Switch(config-if)#exit

Switch(config)#int fa0/9

Switch(config-if)#sw

Switch(config-if)#switchport ac

Switch(config-if)#switchport access vl

Switch(config-if)#switchport access vlan 3

Switch(config-if)#int fa0/10

Switch(config-if)#switchport access vlan 3

Switch(config-if)#int fa0/11

Switch(config-if)#switchport access vlan 3

Switch(config-if)#exit

Switch(config)#int fa0/17

Switch(config-if)#switchport access vlan 4

Switch(config-if)#int fa0/18

Switch(config-if)#switchport access vlan 4

Switch(config-if)#int fa0/19

Switch(config-if)#switchport access vlan 4

Switch(config)#int fa0/1

Switch(config-if)#sw

Switch(config-if)#switchport mo

Switch(config-if)#switchport mode tr

Switch(config-if)#switchport mode trunk

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

Switch(config-if)#exit

Switch(config)#^Z

Switch#

%SYS-5-CONFIG_I: Configured from console by console

wr

Building configuration...

[OK]

Switch#copy ru

Switch#copy running-config st

Switch#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Switch#show vl

Switch#show vlan br

Switch#show vlan brief

2º Configurando o Router com o serviço de DHCP

Router>en

Router>enable

Router#config t

Router#config terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#no ip domain-lookup

Router(config)#ip dh

Router(config)#ip dhcp po

Router(config)#ip dhcp pool ADM

Router(dhcp-config)#net

Router(dhcp-config)#network 192.168.1.0 255.255.255.0

Router(dhcp-config)#de

Router(dhcp-config)#default-router 192.168.1.1

Router(dhcp-config)#dns

Router(dhcp-config)#dns-server 1.1.1.1

Router(dhcp-config)#exit

Router(config)#ip dh

Router(config)#ip dhcp po

Router(config)#ip dhcp pool CONATBIL

Router(dhcp-config)#net

Router(dhcp-config)#network 192.168.2.0 255.255.255.0

Router(dhcp-config)#de

Router(dhcp-config)#default-router 192.168.2.1

Router(dhcp-config)#dns

Router(dhcp-config)#dns-server 2.2.2.2

Router(dhcp-config)#exit

Router(config)#ip dh

Router(config)#ip dhcp po

Router(config)#ip dhcp pool TI

Router(dhcp-config)#net

Router(dhcp-config)#network 192.168.3.0 255.255.255.0

Router(dhcp-config)#de

Router(dhcp-config)#default-router 192.168.3.0

Router(dhcp-config)#dns 3.3.3.3

Router(dhcp-config)#exit

Router(config)#int fa0/0

Router(config-if)#no sh

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#int fa0/0.2

%LINK-5-CHANGED: Interface FastEthernet0/0.2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0.2, changed state to up

Router(config-subif)#en

Router(config-subif)#encapsulation do

Router(config-subif)#encapsulation dot1Q 2

Router(config-subif)#ip ad

Router(config-subif)#ip address 192.168.1.1 255.255.255.0

Router(config-subif)#exit

Router(config)#int fa0/0.9

%LINK-5-CHANGED: Interface FastEthernet0/0.9, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0.9, changed state to up

Router(config-subif)#en

Router(config-subif)#encapsulation do

Router(config-subif)#encapsulation dot1Q 3

Router(config-subif)#ip

Router(config-subif)#ip ad

Router(config-subif)#ip address 192.168.2.1 255.255.255.0

Router(config-subif)#exit

Router(config)#int fa0/0.17

Router(config-subif)#

%LINK-5-CHANGED: Interface FastEthernet0/0.17, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0.17, changed state to up

Router(config-subif)#en

Router(config-subif)#encapsulation do

Router(config-subif)#encapsulation dot1Q 4

Router(config-subif)#ip ad

Router(config-subif)#ip address 192.168.3.1 255.255.255.0

Router(config-subif)#exit

Router(config)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

wr

Building configuration...

[OK]

Router#copy ru

Router#copy running-config st

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#