1º Configurar os Ips dos roteadores e abilitar portas serial e fast

ROTEADOR MCSBC

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface serial 0/0

Router(config-if)#ip address 10.0.0.1 255.0.0.0

Router(config-if)#clock rate 64000

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/0, changed state to down

Router(config-if)#exit

Router(config)#interface fastEthernet 1/0

Router(config-if)#ip address 192.168.1.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet1/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet1/0, changed state to up

Router(config-if)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

ROTEADOR MCSA

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface serial 0/0

Router(config-if)#ip address 10.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/0, changed state to up

Router(config-if)#exit

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0, changed state to up

Router(config)#interface fastEthernet 1/0

Router(config-if)#ip address 192.168.2.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet1/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet1/0, changed state to up

Router(config-if)#exit

Router(config)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#copy ru

Router#copy running-config st

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

2º Habilitar a rota estática entre os roteadores com os ips 10.0.0.1 ao 10.0.0.2

ROTEADOR MCSBC

Router>enable

Router#config terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#ip route 192.168.2.0 255.255.255.0 serial 0/0

Router(config)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#copy ru

Router#copy running-config st

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

ROTEADOR MCSA

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#ip route 192.168.1.0 255.255.255.0 serial 0/0

Router(config)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

3º Colocar ip manual nas maquinas de mcsbc  ip do roteador 192.168.1.1 nas maquinas são 192.168.1.2, 3 e 4 em mcsa ip do roteador e 192.168.2.1 nas maquinas são 192.168.2.2, 3 e 4.

4º Testar usando o ping nas maquinas ou as cartas enviando de mcsbc para mcsa