Configuração Roteador MCSBC

Router>

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#no ip domain-lookup

Router(config)#ip dhcp pool MCSBC

Router(dhcp-config)#network 192.168.0.0 255.255.255.0

Router(dhcp-config)#default-router 192.168.0.1

Router(dhcp-config)#exit

Router(config)#ip dhcp excluded-address 192.168.0.1 192.168.0.50

Router(config)#interface fastEthernet 1/0

Router(config-if)#ip address 192.168.0.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet1/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet1/0, changed state to up

Router(config-if)#exit

Router(config)#interface serial 0/0

Router(config-if)#ip address 10.0.0.1 255.0.0.0

Router(config-if)#clock rate 64000

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/0, changed state to down

Router(config-if)#exit

Router(config)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#wr

Building configuration...

[OK]

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

Configurando a Rota OSPF

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#router ospf 1

Router(config-router)#network 10.0.0.0 0.255.255.255 area 0

Router(config-router)#network 192.168.0.0 0.0.0.255 area 0

Router(config-router)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#wr

Building configuration...

[OK]

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

Router#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

C 10.0.0.0/8 is directly connected, Serial0/0

O 172.16.0.0/16 [110/65] via 10.0.0.2, 00:02:11, Serial0/0

C 192.168.0.0/24 is directly connected, FastEthernet1/0

Configuração Roteador MCSA

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#no ip domain-lookup

Router(config)#ip dhcp pool MCSA

Router(dhcp-config)#network 172.16.0.0 255.255.0.0

Router(dhcp-config)#default-router 172.16.0.1

Router(dhcp-config)#exit

Router(config)#ip dhcp excluded 172.16.0.1 172.16.0.50

Router(config)#interface fastEthernet 1/0

Router(config-if)#ip address 172.16.0.1 255.255.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet1/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet1/0, changed state to up

Router(config-if)#exit

Router(config)#interface serial 0/0

Router(config-if)#ip address 10.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/0, changed state to up

Router(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0, changed state to up

Router(config-if)#^Z  (CTRL + Z)

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#wr

Building configuration...

[OK]

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

Configurando Rota RIP

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#router ospf 1

Router(config-router)#network 10.0.0.0 0.255.255.255 area 0

Router(config-router)#network

02:12:54: %OSPF-5-ADJCHG: Process 1, Nbr 192.168.0.1 on Serial0/0 from LOADING to FULL, Loadi

Router(config-router)#network 172.16.0.0 0.0.255.255 area 0

Router(config-router)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#wr

Building configuration...

[OK]

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

Router#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

C 10.0.0.0/8 is directly connected, Serial0/0

C 172.16.0.0/16 is directly connected, FastEthernet1/0

O 192.168.0.0/24 [110/65] via 10.0.0.1, 00:02:13, Serial0/0